<Product Name> DNSSEC Requirements

| Author: | Ron Santos |
| --- | --- |
| Email: | ron.santos@companyname.com |
| Contributors: | There were 7 contributors: 4 developers, 2 managers, and 1 tester. |

Before using this document, the reader should verify that it is the current version at: Hyperlink to Online Version (This is an uncontrolled document unless viewed online.)

Table of Contents

1.	Feature Identification	1

2.	Feature Description	1

2.1.	Introduction	1

2.2.	Terminology for States of DNSSEC Keys	2

2.3.	Feature Scope	3

2.4.	Functionality	3

2.4.1.	Use Cases	3

2.4.2.	Functional Requirements	6

2.4.2.1.	Product Versions Supported	6

2.4.2.2.	Check if a Child’s Internal Parent Zone is DNSSEC-Enabled	6

2.4.2.3.	KSK and ZSK Rollover Intervals	6

2.4.2.4.	KSK Rollovers Using Double-Signature Method	6

2.4.2.5.	ZSK Rollovers Using Pre-Publish Method	7

2.4.2.6.	Calculating and Limiting TTL Values	7

2.4.2.7.	Email to Send DS Record to Unmanaged Parent Zone	10

2.4.2.8.	View KSKs with Upcoming Rollovers (for Sending DS Records)	11

2.4.2.9.	View and Access Keys in a Zone	11

2.4.2.10.	DS Records in GUI	12

2.4.2.11.	KSK Generation Lead Time Property in Database Only	12

2.4.2.12.	ZSK Generation Lead Time Property	13

2.4.2.13.	Propagation Delay Property	13

2.4.2.14.	DS Registration Delay Property	14

2.4.2.15.	Add New KSK for Automated Key Rollover	14

2.4.2.16.	Delete Old KSK for Automated Key Rollover	15

2.4.2.17.	Add New ZSK for Automated Key Rollover	15

2.4.2.18.	Delete Old ZSK for Automated Key Rollover	15

2.4.2.19.	First Time Signing a Zone	16

2.4.2.20.	Manual KSK Rollover	17

2.4.2.21.	Manual ZSK Rollover	18

2.4.2.22.	Support Multiple Consecutive Manual Key Rollovers	19

2.4.2.23.	Immediate and Scheduled Manual Key Rollovers	20

2.4.2.24.	Status of Long-Running Signing and Re-Signing Processes	20

2.4.2.25.	NSEC3PARAM Values and Functionality	20

2.4.2.26.	Modify Existing Clone DNS Server Function	21

2.4.2.27.	Existing DNSSEC CLI Commands	21

2.4.2.28.	Existing DNSSEC API Calls in <Product Name> Web Service Interface	22

2.4.2.29.	Migration of Existing DNSSEC Zones and Keys	22

2.4.2.30.	Manually Add DS Record to Parent Zone in <Product Name>	23

2.4.2.31.	Log Events During Life of Keys	23

2.4.2.32.	Auditing of DNSSEC-Related User Actions	23

2.4.2.33.	User Documentation	23

2.4.2.34.	Existing CLI Commands and API Calls That Are Not DNSSEC-Specific (Desirable Reqt.)	24

2.4.2.35.	Improve CLI Command to Search for Specific Type of Resource Record (Desirable Reqt.)	25

2.4.2.36.	Deletion of Old Keys from Key Repository (Desirable Reqt.)	25

2.4.2.37.	New DNSSEC CLI Commands (Desirable Reqt.)	25

2.4.2.38.	New DNSSEC API Calls in <Product Name> Web Service Interface (Desirable Reqt.)	26

2.4.2.39.	DNS Server Template Support of DNSSEC Parameters (Desirable Reqt.)	26

2.4.2.40.	Backward Compatibility for Existing DNSSEC Customers is Not Required	26

2.4.2.41.	Support for DNS Views is Not Required	26

2.4.2.42.	DNSSEC Administrator Permissions are Not Required	27

2.4.2.43.	Support for Trust Anchors and Revoking a Key is Not Required	27

2.4.2.44.	Support for RFC-Compliant Key Algorithm Rollovers is Not Required	27

2.4.2.45.	Licensing is Not Required	28

2.5.	Capacity, Performance and Reliability	28

2.6.	Hardware and Software Prerequisites	28

3.	Future Considerations	28

4.	Supporting Data	28

4.1.	Acronyms	28

4.2.	External References	29

4.3.	Revision History	29

5.	Appendix A: KSK Rollover Timelines	33

6.	Appendix B: ZSK Rollover Timelines	34

.

.

.

#### Add New ZSK for Automated Key Rollover

Priority: High

<Begin-DNSSEC-Enh-1400>

During the lifecycle of a DNSSEC-enabled zone, times will occur when a new Zone Signing Key (ZSK) needs to be pre-published in a zone at a specific time before the retirement of the currently active key. This is described in Event 4 on page 11 in section 3.2.1 on “Pre-Publication Method” in RFC 7583 on DNSSEC Key Rollover Timing Considerations [1]. Based on that section, the new key shall be introduced before the retirement date (and time) as per the following formula:

Ipub = Dprp + TTLkey

This formula will be used to calculate what can be called the “ZSK Publication Interval” in <Product Name>, and the following values are used in this formula. (Note that Ipub is defined in RFC 7583 [1] to be the publication interval.)

Dprp: This is the propagation delay time, and the value shall be the value of the Propagation Delay Property (which is described above in requirement DNSSEC-Enh-1000 on this property).

TTLkey: This is the Time to Live (TTL) of the ZSK currently in the zone.

When default values are used in this formula, a new key shall be pre-published 16 (= 1 + 15) days before the retirement of the currently active key. Note that the default rollover interval for a ZSK is 30 days, so for some days there will be 3 keys in a zone: a Retired key, the currently Active key, and a new Pre-Published key.

<End-DNSSEC-Enh-1400>

#### Delete Old ZSK for Automated Key Rollover

Priority: High

<Begin-DNSSEC-Enh-1500>

During the lifecycle of a DNSSEC-enabled zone, times will occur when an old Zone Signing Key (ZSK) that is currently retired needs to be deleted from a zone at a specific time after it changed from being active to being retired. This is described in Event 7 on page 12 in section 3.2.1 on “Pre-Publication Method” in RFC 7583 on DNSSEC Key Rollover Timing Considerations [1]. Based on that section, the old key shall be deleted after the retirement date (and time) as per the following formula:

Iret = Dsgn + Dprp + TTLsig

This formula will be used to calculate what can be called the “ZSK Retirement Interval” in <Product Name>, and the following values are used in this formula. (Note that Iret is defined in the RFC 7583 [1] to be the retire interval.)

Dsgn: This is the signing delay and is the amount of time for all Resource Record (RR) sets in a zone to be signed with a new ZSK. For Dsgn, the value used shall be the Signature Validity period for RRSIG records associated with the new ZSK. (Note that as described in the subsection on “Automatic Key Rollovers” in the BIND 9 Administrator Reference Manual for BIND 9.10.2 [5], BIND normally signs records gradually over the Signature Validity period, and this is done to avoid processing spikes in a DNS server.)

Dprp: This is the propagation delay time, and the value shall be the value of the Propagation Delay Property (which is described above in requirement DNSSEC-Enh-1000 on this property).

TTLsig: This is the maximum Time to Live (TTL) of the RRSIG records that were created with the old key. For ease of determining the TTLsig, the previously calculated Maximum Zone TTL can be used. (As per requirement DNSSEC-Enh-0600 above on “Calculating and Limiting TTL Values,” this maximum was previously calculated to limit the maximum TTLs in a zone. Also, note that as per RFCs 4034 and 4035, the TTL of an RRSIG record must match the TTL of the associated RRset.)

When default values are used in this formula, an old ZSK will be deleted 20 (= 4 + 1 + 15) days after it was retired. Note that the default rollover interval for a ZSK is 30 days, so for some days there will be 3 keys in a zone: a Retired key, the currently Active key, and a new Pre-Published key.

<End-DNSSEC-Enh-1500>

.

.

.

#### Manual ZSK Rollover

Priority: High

<Begin-DNSSEC-Enh-1800>

When a manual Zone Signing Key (ZSK) rollover is done on a zone, <Product Name> will move each ZSK present in the zone one step forward in its timeline. The steps in the timeline are the same as for an automated ZSK rollover, but with a manual rollover the move forward is done immediately instead of according to a previously calculated schedule. (Note that a user could choose to do multiple consecutive manual rollovers to proceed as far as needed in the timeline.) The following functionality shall apply to the ZSK(s) in a zone when a manual rollover is done:

- There shall be a maximum of three ZSKs in a zone: one Retired, one Active, and one Pre-Published.

- There shall be one set of ZSK signatures in a zone (and it will be associated with the Active key).

- The Rollover Interval of ZSKs will remain as configured. (In other words, Rollover Intervals will not be adjusted to compensate for an earlier than usual introduction of a new key.)

- Depending on the number of ZSKs in the zone, the following shall be done:

| Number of ZSKs in Zone | Actions |
| --- | --- |
| 1 | Add a new Pre-Published ZSK to the zone and leave the existing Active ZSK unchanged. |
| 2 | Depending on the types of ZSKs present, do the following: - If one ZSK is Active and the other is Retired, then add a new Pre-Published ZSK and leave the two existing ZSKs unchanged. - If one ZSK is Pre-Published and the other is Active, then retire the current Active ZSK, and change the current Pre-Published ZSK into an Active ZSK. Also, remove the old RRSIG records and sign the RRsets in the zone with the new Active ZSK. |
| 3 | Delete the current Retired ZSK, and leave the current Active ZSK and current Pre-Published ZSK unchanged. |

- If a new Pre-Published ZSK is added, then the scheduled date for its deletion shall be calculated the same way as done for an automated ZSK rollover as specified in requirement DNSSEC-Enh-1500 above on “Delete Old ZSK for Automated Key Rollover.” (Note that dnssec-keygen and dnssec-settime commands support the use of a deletion date.)

Also, in all rollover cases, the following shall be done:

- Early in the rollover process, output a warning message informing the user that performing a manual rollover can break DNSSEC validation by not allowing enough time for a new ZSK to get into resolver caches before it is needed or by prematurely removing a ZSK from a zone during a time when resolvers still need it to validate associated signatures in their caches. Additionally, allow the user to cancel the operation.

- Output a message informing the user of the step(s) that will be done (which depend on the number of keys currently present in the zone and their states) and allow the user to cancel the operation. (On the implementation question of which possible data source should be used to determine the keys in a zone and their states, the master source that is considered authoritative for this information should be used, which probably would be the <Product Name> database.)

- Upcoming tasks or activities that had been originally scheduled in <Product Name> should be rescheduled if needed to take into account the earlier than expected rollover. For example if before the manual rollover was initiated, <Product Name> had a scheduled task to run the dnssec-keygen command in one week to pre-publish a new ZSK with a 30-day life and the user just completed a manual rollover that pre-published a new ZSK with a 30-day life, then the originally scheduled dnssec-keygen command shall be rescheduled to take into account the newly pre-published ZSK. In this case, the pre-publication of the next ZSK should be rescheduled to occur at the correct time before the retirement of the newly pre-published ZSK using the same formula as presented above in requirement DNSSEC-Enh-1400 on “Add New ZSK for Automated Key Rollover.”

<End-DNSSEC-Enh-1800>

.

.

.